|  | Reg. No: Regulation- 2020 (ES) SRI RAMAKRISHNA ENGINEERING COLLEGE,COIMBATORE (Autonomous Institution, Reaccredited by NAAC with ‘A+’ GRADE Approved by AICTE and Permanently affiliated to Anna University, Chennai) AUTONOMOUS EXAMINATIONS - JULY'2022 |
| --- | --- |
| COMMON TO B.E – CSE / B.TECH – AIDS & M.TECH - CSE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20EE262 - BASICS OF ELECTRICAL AND ELECTRONICS ENGINEERING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Answer ALL questions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Duration: 2 Hrs 30 min |  |  |  |  |  | Maximum: 70 Marks |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | PART - A |  |  | (5x 1 = 5 Marks) |  |  |  |
| 1. | CO1 |  | Current through a branch in a linear circuit is 2A when the input voltage is 10V. If the voltage is reduced to 1V and its polarity reversed, the current through the branch will be _________. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | a) -2A |  |  |  |  | b) -0.2A | c) 0.2A |  |  | d) 1A |  |
| 2. | CO1 |  | A 1kW toaster takes roughly 4 minutes to heat four slices of bread. Calculate the cost of operating the toaster once per day for 1 month (30 days). Assume energy costs 4Rs./kWh. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | a) Rs.16 |  |  |  |  | b) Rs.8 | c) Rs.20 |  |  | d) Rs.12 |  |
| 3. | CO3 |  | The basic disadvantage of moving iron instruments is _______. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | a) polarity dependency b) complex construction c) non-linear scaling d) wide frequency applications |  |  |  |  |  |  |  |  |  |  |
| 4. | CO2 |  | The coefficient of coupling for two coils having self-inductances of 2H and 8H with mutual inductance of 3H is _________. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | a) 0.75 |  |  |  |  | b) 0.183 | c) 0.596 |  |  | d) 0.896 |  |
| 5. | CO4 |  | In a common emitter configuration of BJT, the base and collector currents are 50μA and 1mA respectively. Calculate its base amplification factor. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | a) 0.953 |  |  |  |  | b) 0.5 | c) 2 |  |  | d) 20 |  |
|  |  |  |  |  | PART - B |  |  |  |  |  | (5x 3 = 15 Marks) |  |  |
| 6. | CO1 |  | Calculate equivalent resistance across the open terminals. |  |  |  |  |  |  |  |  |  |  |
| 7. | CO1 |  | Apply nodal analysis to find Vo in the circuit |  |  |  |  |  |  |  |  |  |  |
| 8. | CO2 |  | List the characteristics of an Ideal Transformer. |  |  |  |  |  |  |  |  |  |  |
| 9. | CO3 |  | Point out the use of copper shading bands in an induction type energy meter. |  |  |  |  |  |  |  |  |  |  |
| 10. | CO4 |  | Solve for the Zener Current. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | PART - C |  |  | (5 x 10 = 50 Marks) |  |  |  |
|  |  |  |  | Q.No: 11 Compulsory Question Q.No: 12 to 17 Answer any FOUR Questions Compulsory Question: |  |  |  |  |  |  |  |  |  |
| 11. | CO1 |  | i) | Change the given circuit into Thevenin’s Equivalent across terminals ab. Also find the current thru 6Ω resistor. |  |  |  |  |  |  |  |  | (5) |
|  | CO1 |  | ii) | Find the maximum power delivered to the resistor R. |  |  |  |  |  |  |  |  | (5) |
| 12. | CO1 |  | i) | Solve for the voltage across 6Ω resistor using mesh analysis. |  |  |  |  |  |  |  |  | (5) |
|  | CO1 |  | ii) | Calculate the voltage across 1Ω resistor using super position theorem. |  |  |  |  |  |  |  |  | (5) |
| 13. | CO2 |  |  | With neat sketch describe the construction and working principle of a single-phase transformer. Define transformation ratio with related equations. |  |  |  |  |  |  |  |  | (10) |
| 14. | CO3 |  |  | Discuss the working principle of a moving coil instrument. Prove that the deviation made by the pointer is proportional to the current in the coil. |  |  |  |  |  |  |  |  | (10) |
| 15. | CO3 |  |  | Describe two wattmeter method of measuring three phase power. Derive the required equations to prove the total three phase power is sum of the two wattmeter readings. |  |  |  |  |  |  |  |  | (10) |
| 16. | CO4 |  | i) | Arrange a complete power supply circuit using the basic electronic devices. Input is 230V AC and the required output is 5V regulated DC. |  |  |  |  |  |  |  |  | (5) |
|  | CO4 |  | ii) | Compare the different configurations of BJT. Suggest best configurations for voltage, current and power gain. |  |  |  |  |  |  |  |  | (5) |
| 17. | CO4 |  | i) | Explain the operation of a N channel JFET. Derive and explain its transfer characteristics from its drain characteristics with required equations. |  |  |  |  |  |  |  |  | (5) |
|  | CO4 |  | ii) | Discuss on the classification of MOSFET. Also explain the different regions of operation in its output characteristic. |  |  |  |  |  |  |  |  | (5) |
| ***** |  |  |  |  |  |  |  |  |  |  |  |  |  |