(Revised) Regulation- 2020 (ESC)

Reg. No:

# SRI RAMAKRISHNA ENGINEERING COLLEGE, COIMBATORE

(Autonomous Institution, Reaccredited by NAAC with ‘A+’ GRADE Approved by AICTE and Permanently affiliated to Anna University, Chennai)

## AUTONOMOUS EXAMINATIONS - MAY'2024 COMMON TO B.E – CSE / B.TECH – IT / AI&DS &

M.TECH – CSE (5 YEARS INTEGRATED)

## 20EE262 - BASICS OF ELECTRICAL AND ELECTRONICS ENGINEERING

Answer ALL questions

## Duration: 2 Hrs 30 min	Maximum: 70 Marks

PART - A	(5x 1 = 5 Marks)

## CO1	A current of 5A is drawn by a filament of an electric bulb for 2 minutes. Find the amount of electric charge flows through the circuit.

a) 300 C	b) 450 C	c) 600 C	d) 100 C

## CO1	If there are 3 Resistors R1, R2 and R3 in series and V is total voltage and I is total current, then Voltage across R2 is	.

a) VR3/(R1+R2+R3)	b) VR2/(R1+R2+R3)	c) VR1/(R1+R2+R3)	d) V

## CO2	Lamination of the transformer core is made of	.

  - silicon steel	b) cast steel	c) aluminum	d) cast iron

## CO3	When	a	pentavalent	impurity	is	added	to	a	pure	semiconductor,	it becomes		.

  - an insulator	b) an intrinsic semiconductor

c) p-type semiconductor	d) n-type semiconductor

## CO4	The MOSFET is a	device.

  - current controlled	b) voltage controlled

c) frequency controlled	d) power controlled

## PART - B	(5x 3 = 15 Marks)

- CO1	Find the value of current I coming out of the battery for the circuit in the figure?

- CO1	State Kirchoff’s Voltage law and Kirchoff’s current law.

- CO2	Transformers ratings are expressed in kVA. Give reason.

- CO3	BJT can be used as an amplifier -Justify.

- CO4	Define calibration and give reason why it is important for an instrument.

## PART - C	(5 x 10 = 50 Marks)

Q.No: 11 Compulsory Question Q.No: 12 to 17 Answer any FOUR Questions

Compulsory Question:

- CO1	i)	Determine the equivalent resistance across the terminals AB for the circuit.

(5)

CO1   ii)	Determine the mesh currents in the circuit using mesh analysis.	(5)

| 12. | CO1 |  | Determine the current through the 5Ω resistor in the circuit shown in Fig.4 using Thevenin’s theorem. | (10) |
| --- | --- | --- | --- | --- |
| 13. | CO2 |  | Outline the construction of single phase transformer and explain the working principle of transformer. | (10) |
| 14. | CO3 | i) | Outline the functional elements of an instrument with a neat diagram. | (5) |
|  | CO3 | ii) | Discuss the working principle of a moving coil instrument. | (5) |
| 15. | CO3 |  | Explain the methods used for measurement of three phase power in three phase circuits. | (10) |
| 16. | CO4 |  | Explain the construction, working, Characteristics, and applications of PN junction diode. | (10) |
| 17. | CO4 | i) | With a neat sketch, explain the working of JFET. | (5) |
|  | CO4 | ii) | Explain the different regions of operations in the output characteristics of MOSFET. | (5) |
|  |  |  | ***** |  |