|  | Reg. No: Regulation- 2020 (ES) SRI RAMAKRISHNA ENGINEERING COLLEGE,COIMBATORE (Autonomous Institution, Reaccredited by NAAC with ‘A+’ GRADE Approved by AICTE and Permanently affiliated to Anna University, Chennai) AUTONOMOUS EXAMINATIONS - AUGUST'2022 |
| --- | --- |
| B.TECH - INFORMATION TECHNOLOGY |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20EE262 - BASICS OF ELECTRICAL AND ELECTRONICS ENGINEERING |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Answer ALL questions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Duration: 2 Hrs 30 min |  |  |  |  |  | Maximum: 70 Marks |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | PART - A |  |  | (5 x 1 = 5 Marks) |  |  |
| 1. | A circuit consists of two resistances, R1 and R2 in parallel. The total current passing through the circuit is IT. The current passing through R1 is ______. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | a) (ITR1)/(R1+R2) |  |  |  | b) IT(R1+ R2)/ R1 |  |  |  | c) (ITR2)/(R1+R2) |  |  | d) (ITR1+ R2)/ R2 |  |
| 2. | Maximum power is transferred when load resistance is________. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | a) equal to half of the source resistance b) Equal to zero c) Equal to source resistance d) unity |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. | The reluctance of magnetic circuit is defined as _________. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | a) Magneto motive force b) ratio of magneto motive force to magnetic flux c) ratio of magnetic flux to magnetic force d) ratio of voltage to current |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. | The direction of induced e.m.f. in a coil can be found by _________. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | a) Laplace's law b) Lenz's law c) Fleming's right hand rule d) Kirchhoff's voltage law |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. | A moving iron type ammeter has few turns of thick wire so that ___. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | a) Resistance is less b) Sensitivity is high c) Damping is effective d) Scale is large |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | PART - B |  |  | (5 x 3 = 15 Marks) |  |  |  |
| 6. |  | State Kirchhoff’s voltage and current law. |  |  |  |  |  |  |  |  |  |  |  |
| 7. |  | Find the value of current I coming out of the battery for the circuit in the figure 1? Fig.1 |  |  |  |  |  |  |  |  |  |  |  |
| 8. |  | Distinguish between self and mutual inductance. |  |  |  |  |  |  |  |  |  |  |  |
| 9. |  | Why calibration of instrument is important? |  |  |  |  |  |  |  |  |  |  |  |
| 10. |  | Explain the operation of NPN and PNP transistors. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | PART - C |  |  | (5 x 10 = 50 Marks) |  |  |
|  |  |  |  | Q.No: 11 Compulsory Question Q.No: 12 to 17 Answer any FOUR Questions Compulsory Question: |  |  |  |  |  |  |  |  |  |
| 11. |  |  | i) | Calculate the Thevenin’s voltage, Thevenin’s resistance and the load current. Consider the load resistor RL is 40 Ω and ETH is 10 V and Thevenin’s Equivalent Circuit for figure 2. Fig. 2 |  |  |  |  |  |  |  |  | (5) |
|  |  |  | ii) | Determine the Norton’s current for the given circuit across the load resistance RL shown in the figure 3. Fig. 3 |  |  |  |  |  |  |  |  | (5) |
| 12. |  |  | i) | Apply Superposition theorem, calculate current flowing through 2 Ω resistor shown in the figure 4. Fig. 4 |  |  |  |  |  |  |  |  | (5) |
|  |  |  | ii) | State Maximum power transfer theorem. Apply the specified condition and derive the open circuit voltage, thevenin’s resistance and maximum power transferred. |  |  |  |  |  |  |  |  | (5) |
| 13. |  |  |  | With a neat sketch, Explain the construction, working, types, applications and advantages of single phase transformer. |  |  |  |  |  |  |  |  | (10) |
| 14. |  |  |  | Classify the types of measuring instruments and explain briefly. |  |  |  |  |  |  |  |  | (10) |
| 15. |  |  |  | Explain the construction, operation, merits and demerits of Permanent Magnet Moving Coil instrument. |  |  |  |  |  |  |  |  | (10) |
| 16. |  |  |  | Explain the construction, working, VI characteristics, applications and advantages of PN junction diode. |  |  |  |  |  |  |  |  | (10) |
| 17. |  |  |  | With a neat sketch, explain the construction, principle and working of JFET. Give their merits, demerits and applications. |  |  |  |  |  |  |  |  | (10) |
| ***** |  |  |  |  |  |  |  |  |  |  |  |  |  |